-*- Mode: indented-text -*-

--- $Id: scsiopt.doc,v 1.4 1998/03/04 02:24:32 vitus Exp $

SCSI Optimizer - Driver Dokumentation

DESCRIPTION

SCSIOpt is a simple OS/2 filter driver to modify some performance

related mode pages of your SCSI devices.

Source code  is included to serve  as  a sample how  to  write a

16bit driver.

LEGAL STUFF (we need this, don't we?)

The author  disclaims all warranties  as to this software, whether

express  or  implied,  including  without limitation  any  implied

warranties of  merchantability, fitness for  a particular purpose,

functionality,  data  integrity  or   protection,   in so  far  as

permitted by applicable legislation.

All trademarks are property of their respective owners and appear

for identification purposes only.

There is a copyright  on some  source code  files in  this archive

(cmd* files) but as they are freely distributed via IBM's webpages

I don't think this is a problem.  All other sources are put to the

public domain in the hope that they may be usefull.

CAUTION

Modifying mode  pages on SCSI devices is  a very low-level action.

Although I have  never encountered a  totally useless device  when

experimenting with SCSIOpt.flt you may end up reading strange data

(which in turn may lock up your  OS/2).  For this reason I decided

that this driver  will never save a mode  page permanently.   Just

power cycle your machine, remove  SCSIOpt.flt from your config.sys

(booting from  floppy if neccessary) and  you are back to what you

had before trying SCSIOpt.flt.

INSTALLATION

It is recommended you  have an OS/2 boot disk  ready, just in case

the system locks up during the boot process and you want to remove

the driver.  On Warp you may use ALT-F1, C for the same purpose to

boot to a command line with your orginal Config.Sys.

Execute DDINSTAL from a  OS/2 command  prompt or double-click  the

Device Driver Install object  (located in the System Setup folder)

if your system is Warp.  Change the source directory if neccessary

and a  selection list will  open asking you   to install the 'SCSI

Optimizer'.  Mark that line and click on OK.

Installation by hand is easy:

Copy  SCSIOpt.flt to c:\ or  c:\os2\boot, assuming c: is your

boot drive.

Edit  your  c:\Config.Sys file (i.e.  using  the  OS/2 "System

Editor"):

Add the following line to your Config.Sys:

BASEDEV=SCSIOpt.flt /V		(no subdirectory name!)

The  position   of  'BASEDEV=SCSIOpt.flt' in  Config.Sys  doesn't

matter.  If, however, you encounter  problems try top of file  (if

this  solves  your problems please report  with  a listing of your

Config.Sys).

After installation this driver will do nothing but search for SCSI

devices and display  their  coordinates.  This  will allow you  to

select the devices you might be able to modify.

No  modifications  are actually done.   See  below  how  to modify

pages.

OPTIONS

Two general options, i.e. option which have to appear before any

'/A:n' option:

/V

Verbose, display some usefull and unusefull information (this

switch is enabled in automatic installations).

Without this switch the driver will be quiet except for fatal

errors.

/DEBUG

Debugging display, a lot of.

One  word  about screen output:    BASEDEV drivers cannot  display

anything   on the console  immediately,  they queue messages to be

displayed later by  the  system initialization.  Due to  this fact

you might not   see any  messages at  all  (if system  hangs/traps

before they are yanked to console).

Best chances to see anything is to copy  this driver to the second

of your  "utility disks" (or installation  disks  if you don't run

Warp), change  the Config.Sys there (BASEDEV=SCSIOpt.flt /DEBUG

and boot from them.  Now you don't rely on any SCSI device.

If you get  a  SYS2068 at  boot time,  all messages  generated  by

BASEDEV    drivers exceed a  kernel limit   and nothing  at all is

displayed.  Ignore  that  error and   use SCSIOpt.exe  to  display

SCSIOpt.flt messages or change options to display less.

Adapter selection option:

/A:n

'n' is an index, it depends on the ADD order in your

Config.Sys and numbers all adapter (including floppy

controllers, etc.).

Unit control options (used after /A:n):

/E:(u1,page1,mask1),(u1,page2,mask2),...

/!E:(u1,page1,mask1),(u2,page2,mask2),...

'u' is  an unit index which  the  ADD determines.   BTW: these

values  are  displayed by   SCSIOpt.flt during  boot if /V  or

/DEBUG option is provided.

These options allow you to specify which pages to modify.  Use

'/E:' to enable   features and '/!E:' to   disable them.   Any

combination  is valid, the last  option of conflicting options

will survive.

MODE PAGE OPTIONS

As the general layout of a mode page option is (<unit index>,<page

number>,<mask>) you have three values to supply:

<unit index>	this value depends on the ADD which controls

a unit.  Enable SCSIOpt's option '/V' and you

will see messages telling what indexes were

found.

<page number>	tells SCSIOpt which page to change

<mask>		tells SCSIOpt which features of a mode page to

enable/disable.  This is a bit-coded value

(but decimal...).

page	bit(2^x) description (enable/disable ...)

2	0	... disconnect because of empty (read) or full

(write) buffers

2	1	... inactive bus timeout

2	2	... disconnect timeout

2	3	... connect timeout

8	0	... write cache

8	1	... read cache

8	2	... prefetch

8	3	... higher importance of prefected data than

regular data

10	0	... tagged queuing

10	1	... unrestricted command reordering

13	<all>	... seconds until CD-ROM stops spinning (up to

1920 supported (= 32 minutes)) [this is *not*

bit-coded!]

Hint: if you set 'page'  to a supported value  and 'mask' to  zero

SCSIOpt   will read  that page and    test whether it has  SCSI-II

layout.  If you add '/V' to your command line  it will display the

current settings, too!

SUPPORT PROGRAMS

SCSIOpt.flt   contains    a simple   IOCtl   interface   to  allow

applications to retrieve  messages displayed at  boot time.  It is

described in dskpfpub.h and used by SCSIOpt.exe.

Just execute SCSIOpt without parameters  and it will display those

messages.  Very simple.

TESTED EQUIPMENT

I added  a list of option which  were tested with my devices here.

Keep in mind that in SCSI many things are free to support, so even

with a newer device you might have less options to change.

If  you happen to  know a vendor unique mode  page  that you think

should  be  modified: please mail vendor  string,  device and mode

page description.

Western Digital - Piranha SP4200

Disconnect-reconnect page(2)

buffer full/empty	not supported

bus inactive	not supported

Cache Page(8)

Read Cache Disable	supported

Write Cache Enable	not supported

pre-fetch		not supported

priority		not supported

Control Page(10)	- cannot be read

IBM DORS-32160

Disconnect-reconnect page(2)

buffer full/empty		supported

bus inactive timeout	not supported

connection timeout		not supported

disconnection timeout	not supported

Cache Page(8)

Read Cache Disable	supported

Write Cache Enable	supported

pre-fetch		supported

priority		not supported

Control Page(10)

Algorithm		unrestricted and restricted supported

tagged queuing	not tested

Sony CDU-615S

Disconnect-reconnect page(2)

buffer full		not supported

bus inactive	not supported (device ignores new values)

Cache Page(8)		- cannot be read

Control Page(10)	- cannot be read

CDROM Page(13)		- supported

Tandberg TDC-3660

Disconnect-reconnect page(2)	- cannot be read

Seagate WREN Runner 7 (Imprimis)

Layout of mode pages isn't SCSI-II compatible.

TODO LIST

- Ideas from YOU?

- remove SCSIOpt.flt   from memory  to save  some   KB of  memory.

Perhaps  switched on or   off  with a parameter  (/V  -  keep in

memory?).

- Keep SCSIOpt.flt  in  memory and write a  simple  .EXE to change

values at any  time.  But this is the   same I already  did with

ASPI Router for OS/2  and scache.exe (see aspi01.zip or higher).

Pro:  it would  demonstrate  a complete  filter; Con: it's  more

dangerous to change at runtime as at  boot time (a SCSI bus hang

or reset affects all devices on that bus).

- easier user  interface to select which pages  and how to modify.

I'm most certainly not satisfied with the current solution.

- Use Daniela Engert's description to make  this driver full 32bit

without  mwdd32.zip.   Nice   exercise and would   support  more

compilers.

COMPILING

What do you need?

- IBM's DDK for OS/2:

available     from   http://service.boulder.ibm.com/ddk/ after

registration at no charge.

- a 16bit C compiler

supported out of the box  are Micro Soft C  5.1 - 7.0 (request

C7OS2.ZIP to get those OS/2 files  Micro Soft removed from the

7.0 release) and WatCom C 10.0 and higher.

IMHO Visual C 1.x should work, too; but you would have to use

a MDOS session.

You should be able to use any other 16bit compiler for OS/2

or DOS, but would have to make the modifications yourself.

- a 16bit assembler

the DDK  ships with  MASM 5.1 and  ALP.  Makefiles  are mostly

written for MASM, but TASM or (brrr) WASM should be possible.

It's  only a  single file  if you  don't  plan to  rebuild the

support libraries supplied by the DDK.

- DMake

DMake    3.8  and  4.0   are   available  as  dmake38.zip  and

dmake40o.zip.  Porting to a GNU Make compatible tool shouldn't

be too much work.

Actually compiling

- setup your compiler (PATH and INCLUDE files)

- extract long arithmetik support routines (see Makefile)

- edit Makefile (path to DDK)

- type 'DMake debug all'

Debugging

- Debugging device  drivers is documented  in  the DDK.   It's not

complicated but you need some time to get used to it, especially

if you  plan  to  use a  simple   terminal (as I  like   to do).

Expirience with DEBUG or SYMDEB is helpfull.

AUTHOR

Vitus Jensen

Address: Tischlerstr. 8, D-30916 Isernhagen, Germany

Fidonet: 2:2474/424, OS2Net, TeamOS/2 Net

Usenet: Vitus_Jensen@teaparty.fido.de

BBS: +49-5136-893003 (X.75, V32bis)

FAX: +49-5136-893003

Music: J.S.Bach Orchestersuiten, J.S.Bach Goldberg-Variationen

Written for Version: 0.71

Date: 03.03.1998

Other products: dsksl*.zip, dtest*.zip, aspio*.zip

=== The End ===