# C168H/PCI Document

1.MOXA Vendor ID = 0x1393

2.C168H/PCI Device ID = 0x1680

3.Hardware resource on Device Configuration register of PCI configuration space:

- IRQ 	0x3C, BYTE

- Interrupt Vector	0x1C, DWORD 
Bit 0 : 	Always 1
Bit 1 : 	Reserved
Bits 31: 2 : 	Interrupt Vector

- I/O Base Address	0x18, DWORD 
Bit 0 : 	Always 1
Bit 1 : 	Reserved
Bits 31: 2 : 	I/O Base Address

- Using NT Hal function to get C168H/PCI resource: 
Please see MoxaFindPCIBoards() of initunlo.c in driver source code