KERMIT C-64, v1.5

Robert W. Detenbeck, University of Vermont.

KERMIT is a protocol for transferring files between different

computers.  This KERMIT program was written in the FORTH language, for

a Commodore C64 computer with a Commodore 1541 disk drive (device 8)

and a Commodore 1600 modem on the RS-232 port (device 2).  Other

drives and modems may work if they use the same device numbers.  The

program itself uses about 30K; disk buffers for "save" and "help"

commands extend downward from $9FFF; and there are scratch string

areas in $CB00-$CFFF.  This program has been tested only under limited

conditions.  It has been run at 300 baud, but the nature of the

interrupt-driven RS-232 interface suggests that 1200 or even 2400 baud

should work without loss of data, though perhaps not at the efficiency

of a machine-language program.  Multiple-file transfers with wild-card

construction cause some problems in the transfer of filenames from

Kermit-20.

The "help" files, named scr96, scr97 and scr98, give details of

this program's special characteristics.  Several special features were

required by the C64's use of Commodore's own form of ASCII code, and

its lack of certain ASCII capabilities.  ASCII backslash prints as a

vertical bar, chr$(221), but is transmitted by the "pound" key; the

tab and formfeed characters are stored in C64 files as chr$(220) and

chr$(219).

The KERMIT "connect" command converts the C64 into a simple

terminal, in which the C64 "delete" button transmits the ASCII code

127, "DEL".  If the host mainframe requires a backspace, ASCII code 8,

the F5 key should be used.  The terminal program contained herein

responds correctly to a backspace character received from the modem.

You have received a "turnkey" copy of the FORTH program, in which

the source screens and programming system are inaccessible.  This copy

may be recopied and distributed without restriction.  In fact, the

"new" command of this KERMIT facilitates such copying.  If you want a

copy of the original FORTH screens, with a printing program to device

4, you may obtain one by sending a blank disk in a stamped,

self-addressed mailer to the author, Robert W. Detenbeck, Department

of Physics, University of Vermont, Burlington, VT 05405.  They were

written to be used with version A of C64-FORTH, sold by Performance

Micro Products, 770 Dedham Street-S2, Canton, MA 02021.  Slight

modifications would be required to use the screens with the newer

version B, a pure FORTH-79 standard.  Because the screens of C64-FORTH

are 25 x 40, extensive retyping would be necessary to format them for

readability on a standard 16 x 64 FORTH screen.

Information about KERMIT can be obtained from the following

sources:

Frank da Cruz and Bill Catchings, "Kermit: A File-Transfer Protocol

for Universities"

BYTE, vol. 9, no. 6, June 1984, p. 255 (Part 1)

BYTE, vol. 9, no. 7, July 1984, p. 143 (Part 2)

KERMIT USERS GUIDE, 4th ed., available for $5.00 from KERMIT

Distribution, Columbia University Center for Computing Activities, 7th

Floor, Watson Laboratory, 612 West 115th Street, New York, N.Y. 10025.

For those near the University of Vermont, the USERS GUIDE can be

purchased from the Academic Computing Center in the Cook Building.