ESS AudioDrive Device Nodes

Version 2.05

Date: February 3, 1997

Devnode	Chip	Bustype	Notes

ESS4881	488	Legacy

ESS6881	688	Legacy

ESS8881	888	Legacy

ESS1481	1488	Legacy

ESS1681	1688	Legacy

ESS1781	1788	Legacy

ESS1882	1887	Legacy

ESS1881	1888	Legacy

ESS1691	1698	Legacy

ESS1861	1869B	Legacy

ESS6880	688	Legacy	Multifunction device (ESS6881+PNPB02F)

ESS1680	1688	Legacy	Multifunction device (ESS1681+PNPB02F)

ESS0100	688	BIOSPnP

ESS0108	888	BIOSPnP

ESS0102	1688	BIOSPnP

ESS0104	1788	BIOSPnP

ESS0117         1698   BIOSPnp

ESS0107	1887	BIOSPnP

ESS0106	1888	BIOSPnP

ESS0109	1869B	BIOSPnP

ESS0112	1688	BIOSPnP	Joystick + "ESS0102 equivalent"

ESS0114	1788	BIOSPnP	Joystick + "ESS0104 equivalent"

ESS0116	1888	BIOSPnP	Joystick + "ESS0106 equivalent"

ESS0118         1698    BIOSPnP	Joystick + "ESS0117 equivalent"

ESS0119         1698    BIOSPnP	Joystick + "ESS0109 equivalent"

ESS0101	688	BIOSPnP	DO NOT USE! (was for 'no DMA' designs)

ESS0103	1688	BIOSPnP	DO NOT USE! (was for 'no DMA' designs)

ESS0105	1788	BIOSPnP	DO NOT USE! (was for 'no DMA' designs)

ESS0001			generic game port

ESS0002			generic ide interface

ESS0008			generic MPU-401

ESS0000	1868	ISAPnP	1868 control interface

ESS0003	1868	ISAPnP	1868 fax/modem

ESS1868	1868	ISAPnP	1868 audio/FM/MPU-401

ESS0006	1869	ISAPnP	1869 control interface

ESS0007	1869	ISAPnP	1869 audio/FM/MPU-401/No_spatializer

ESS1869	1869	ISAPnP	1869 audio/FM/MPU-401/Spatializer

ESS0004	1878	ISAPnP	1878 control interface

ESS0005	1878	ISAPnP	1878 digital game port

ESS1878	1878	ISAPnP	1878 audio/FM/MPU-401

ESS0968	968 + 1688	ISAPnP	Wave is PnP, FM and MPU-401 are legacy

ESS2708	1868+2708	ISAPnP	ES2708 EDSP Modem/ES1868 PnP rev C

ESS2000	2708	ISAPnP	ES2708 EDSP Modem/National PnP

ESS2001	2708	ISAPnP	ES2708 EDSP Modem/ES1868 PnP rev E

ESS2810	2810	ISAPnP	ES2810 EDSP external ROM

ESS2080	2810	ISAPnP	ES2810 EDSP internal ROM

ESS2820	2820	ISAPnP	ES2820 EDSP development

ESS2002	2820	ISAPnP	ES2820 EDSP Modem Rev A

ESS2003	2820	ISAPnP	ES2820 EDSP Modem/ES1868 PnP rev A